| Test Procedure | ADZS-AD2428MINI |
| --- | --- |

Document No.:	18- 057361-01 Rev A

Title:	ADZS-AD2428MINI Customer Evaluation Board Test    Procedure

| REVISION HISTORY |  |  |  |  |
| --- | --- | --- | --- | --- |
| Revision | ECR # | Description of Change | Date | Author |
| A | ECR-????? | Initial Release | 09/10/19 | David Harrington |

| Required Approvers |  |
| --- | --- |
| Approver Roles | Approver Names |
| Apps Engineer |  |

ADZS-AD2428MINI Testing Procedure

| Part Number: | ADZS-AD2428MINI | Description: | AD2428 Mini Module evaluation board |
| --- | --- | --- | --- |
| Date: | 11/27/19 | Written by: | David Harrington |
| Test Fixture: | Laptop | SW – Initial/Date: |  |
| Approved by: |  | HW – Initial/Date: |  |
| Computer: |  |  |  |

- Document Revision History

| Revision Number | Date | Description | Engineer |
| --- | --- | --- | --- |
|  | 11/27/19 | Initial release | David |

- Required Equipment

  - 3ft USB 2.0 A MALE TO MICRO-USB B MALE CABLE C2G P/N:27364

  - 3ft USB 2.0 A MALE TO MINI-USB B MALE CABLE

  - ADZS-21569-SOM

  - ADZS-EZKIT-SOM

  - 12v Power Supply

  - Duraclick A2B Cable

  - Two “Golden” ADZS-AD2428MINI Modules (1 Master & 1 Bus Power)

- Required software/files

  - ADZS-21569 Baseboard POST.zip file

  - ADI CCES version 2.9.0 or later

  - Termite Terminal Emulator

- Setup

  - 1 Switch and Jumper Settings

•	Boot mode switch (SW1) is set to 1 on ADZS-21569-SOM

•	JP1 Pin 2&3 on ADZS-21569-SOM

•	Set SW1 on ADZS-EZKIT-SOM to all ON to Enable ICE

•	ADZS-EZKIT-SOM P4 pins 1&2, 3&4, 5&6 and 7&8	shorted

•	P13 on ADZS-EZKIT-SOM should have jumper to Pin 1&3 and Pin 2&4

  - 2   Connections

- Connect Micro USB cable between P2 on ADZS-21569-SOM and laptop.

- Connect Mini USB cable between P11 on ADZS-EZKIT-SOM and laptop.

- Programmed Parts

  - Check BOM Revision

Ensure that the BOM REV is the latest.

  - Programming Serial Flash – U11 (ADZS-21569-SOM)

- Now proceed to the test machine that will be used to test the board.

- Extract all of the files from the previously installed “manufacturing” zip folder to the desktop.

- Make sure the boot mode switch (SW1) is set to 1.

- Connect a Mini USB cable from the PC to the USB Embedded ICE

- Connect a Micro USB cable from the PC to the USB TO UART connector on the DSP SoM

- To load Serial flash (on DSP SoM) with

  - Navigate to the “POST” folder.

  - Double click on the 21569_POST_baseboard.bat file.

  - Make sure there were no errors during programming.

  - Press any key to exit.

- Open a Termite terminal session and configure the serial terminal with the following settings: COMxxx (Not COM3), 9600 BAUD, Data bits: 8, Parity: none, Stop bits: 1, Flow control: None, Append LF selected, Font: monospaced

- Testing:

WARNING: Please perform the tests in the order described here to avoid possible damage to the hardware and/or unpredictable results.

Be sure boot mode switch (SW1) is set to 1.

The ADZS-AD2428MINI has two configurations that will need to be tested: Master Mode

Bus Power Mode

Each Module will need to be tested in each configuration.

All tests use the same Test 16 in the POST software, what differs is the Jumper configurations on the ADZS-AD2428MINI module and which connector on the ADZS-EZKIT-SOM it’s connected to.

ADZS-AD2428MINI Jumper Setup

Master Mode:

P3	Pin 2&3

P4	Pin 2&3

P5	Jumper Removed

P6	Pin 2&3

P7	Jumper Removed

P8 	Pin 1&2

Bus Power Mode:

P3	Pin 2&3

P4	Pin 2&3

P5	Pin 1&2

P6	Pin 1&2

P7	Pin 1&2

P8 	Jumper Removed

Tests:

A2B Master Mode & Bus Power Mode Test

- Connect the “Golden” ADZS-AD2428MINI module configured for Master mode and plug it onto J10 on the ADZS-EZKIT-SOM.

- Configure the jumpers on the DUT ADZS-AD2428MINI module for Bus Power Mode and do not plug it onto any board.

- Connect Duraclick cable from P10 on the Master module to P9 on the Bus Power module

- Plug in power to the ADZS-EZKIT-SOM and in the Termite window run test # 16.

- The Master Mode passes if DS1 and DS3 are lit up, and DS2 is off.

- The Bus Power Mode passes if DS1, DS2 and DS3 are lit up.

- If DS2 on the Master is lit, that means either the Master or Slave has failed. Check all jumper settings on all boards and the A2B cable hookup and repeat test. If still fails, change out Slave node and try again.

- Unplug power from the ADZS-EZKIT-SOM.

- Remove the “Golden” ADZS-AD2428MINI module configured for Master mode from J10 on the ADZS-EZKIT-SOM.

- Configure the jumpers on the DUT ADZS-AD2428MINI module for Master Mode and plug it onto J10 on the ADZS-EZKIT-SOM.

- Connect Duraclick cable from P10 on the DUT Master module to P9 on the “Golden” Bus Power module.

NOTE: Do Not Plug “Golden” Bus Power module into any board.

- Plug in power to the ADZS-EZKIT-SOM and in the Termite window run test # 16.

- The Master Mode passes if DS1 and DS3 are lit up, and DS2 is off.

- The Bus Power Mode passes if DS1, DS2 and DS3 are lit up.

- If DS2 on the Master is lit, that means either the Master or Slave has failed. Check all jumper settings on all boards and the A2B cable hookup and repeat test. If still fails, change out Slave node and try again.

- Unplug power from the ADZS-EZKIT-SOM.

THIS CONCLUDES THE TESTING.