Changes required to the SRC code

The modifications, which have to be done for SRC, are as follows.

- In the include file SRC_inc.h, two new elements int  *in_c  & int  *out_c should be added to the structures FIRST_STAGE_ENTRY, SEC_STAGE_ENTRY and THIRD_STAGE_ENTRY. The element *in_c is for the base address of the input in each stage. The element *out_c is for the base address of the output in each stage.

For example, the first stage structure will be as follows-

typedef struct {

int *in_s;		// input signal buffer

int	in_z;		// input signal buffer size

int *out_s;		// output signal buffer

int	out_z;		// output signal buffer size

int *h;			// filter

int plen;		// filter polyphase length

int	up;			// interpolation factor

int	dn;			// decimatation factor

int	nis;		// number of inputs

int	nos;		// number of outputs

int	nshft;		// number of shifts

int	*in_c;		/* input signal */

int	*out_c;		/* output signal*/

int	space;		// nop

} FIRST_STAGE_ENTRY;  // (nos * dn) = (nis * up) is required

- In the include file src_48to441.h, the definition of DOFS3 should be changed from NOUTS-TOFS3 to NOUTS i.e. now it should be #define DOFS3 NOUTS.

- In src.c, initialize the included new elements of three structures as follows.

In init_first_stage(), for the first stage

V  in_c = &in1 [0];

V  out_c = &in2 [0];

In init_sec_stage(), for the second stage

M  in_c = &in2 [0];

M  out_c = &in3 [0];

In init_last_stage(), for the last stage

L  in_c = &in3 [0];

L  out_c = &in4 [0];

- The required changes in the file src_flt.asm are as follows.

  - While copying the input to the buffer in1[], the base address b0 should not be equal to i0. b0 should be equal to V  in_c.

The following code

R6 = [p0++];		// r6 -> first input buffer 'inx'

I0 = r6;		// i0 -> first input buffer 'inx'

B0 = r6;		// b0 -> base of first input circular buffer

is changed to

P5 = 44;

R6 = [p0++p5];	// r6 -> first input buffer 'inx'

I0 = r6;		// i0 -> first input buffer 'inx'

P5=-40;

R6 = [p0++p5];

B0 = r6;		// b0 -> base of first input circular buffer

  - In the upstage, the base addresses of input and output b3 & b2 should be equal to in_c and out_c respectively of corresponding structure.

For input,

the code

R7 = [p0++];		// r7 -> input signal 'inx'

B3 = r7;		// b3 set for circular buffering

is changed to

P4 = 44;

R7 = [p0++p4];	// r7 -> input signal 'inx'

P4 = -40;

R5 = [p0++p4];

B3 = r5;		// b3 set for circular buffering

For output,

the code

R6 = [p0++];

I2 = r6;	// i2 -> output signal 'inx'+1 buffer (output buffer)

B2 = r6;	// b2 set for circular buffering

is changed to

P4 = 40;

R6 = [p0++p4];

I2 = r6;	// i2 -> output signal 'inx'+1 buffer (output buffer)

p4 = -36;

r6 = [p0++p4];

B2 = r6;		// b2 set for circular buffering

  - In the pivot stage, the base addresses of input and output b3&b2 should be fetched from the structure elements in_c and out_c of corresponding structure.

For input,

The code

R7 = [p0++];			// r7 - > input signal ('in*' buffer)

I3 = r7;			// i3 - > input signal

B3 = r7;			// b3 set for circular buffering

is changed to

P4 = 44;

R7 = [p0++p4];		// r7 -> input signal 'inx'

I3 = r7;

P4 = -40;

R5 = [p0++p4];

B3 = r5;

For output,

the code

R6 = [p0++];

I2 = r6;	// i2 -> output signal 'inx'+1 buffer (output buffer)

B2 = r6;	// b2 set for circular buffering

is changed to

P4 = 40;

R6 = [p0++p4];

I2 = r6;	// i2 -> output signal 'inx'+1 buffer (output buffer)

P4 = -36;

R6 = [p0++p4];

B2 = r6;		// b2 set for circular buffering

  - In the down stage also the base addresses of input and output b3&b2 should be fetched from the structure in_c and out_c of corresponding structure.

For input,

the code

R7 = [p0++];		// r7 -> input signal 'inx'

I3 = r7;		// i3 -> input signal 'inx'

B3 = r7;		// b3 set for circular buffering

is changed to

P4 = 44;

R7 = [p0++p4];	// r7 -> input signal 'inx'

I3 = r7;

P4 = -40;

R5 = [p0++p4];

B3 = r5;

For output,

the code

R6 = [p0++];

I2 = r6;	// i2 -> output signal 'inx'+1 buffer (output buffer)

B2 = r6;	// b2 set for circular buffering

is changed to

P4 = 40;

R6 = [p0++p4];

I2 = r6;		// i2 -> output signal 'inx'+1 buffer (output buffer)

P4 = -36;

R6 = [p0++p4];

B2 = r6;		// b2 set for circular buffering

  - The shift operation A1 = A1>>16 is changed to A1 = A1>>>15 at all places involving 32 bit multiplications.

Note: The changes and comments that are done to the original code (Rev. 0.2) are indicated as “IPDC comments” or “IPDC additions” in the supplied project.