Measuring TCP/IP Performance With ADI-TTCP

Colin McPhail

Edinburgh Design Centre

| Issue | Date | Initials | Comments |
| --- | --- | --- | --- |
| Draft 1 | 29 Aug, 2005 | CMcP | Initial issue. |
| Issue 1.0 | 13 Oct 2005 | CMcP | Corresponds to rev 1.0 of the ADI-TTCP program. |

Abstract

Analog Devices has modified PCATTCP, a freely-available program for testing and benchmarking TCP/IP stack implementations, to make it more suited to automated or unattended testing use. The resulting program, called ADI-TTCP, can be built to run on a PC under Windows/winsock2 or on a BF537 EZ-Kit under VDK/lwIP. Porting to another processor and TCP/IP stack should be relatively easy if the stack provides a standard BSD-style ‘sockets’ API.

This note gives an overview of the ADI-TTCP program and describes how to deploy and control instances of it in order to obtain throughput and efficiency measurements.

# Introduction

The ttcp program has been available for more than twenty years on Unix-like systems as a tool for testing TCP and UDP network functionality and obtaining throughput figures. There have been many improvements made to it over this time and many ports to other operating systems. One of the most recent ports to Windows operating systems (specifically, to those supporting the winsock2 network library) was made by the Printing Communications Assoc., Inc. (PCA). This port is known as PCATTCP and its source is made freely available on the PCA website. See Appendix A for the terms and conditions under which PCA makes the program available.

PCATTCP acts as the source or sink of data transferred via a TCP stream or UDP datagrams. Command line arguments specify the amount of data to be transferred, the ‘record size’ to be used, a desired transmit rate, the IP address of the other participant, and so on. Upon completion it prints a throughput rate for its end of the transfer (that is, the number of bytes transmitted or received per second). Some of PCATTCP’s features are particularly useful, notably the ability to regulate the rate at which it transmits data, but for use in embedded systems it lacks some essential capabilities such as being able to measure processor load. Also, the fact that it requires to be reloaded for each change in parameters makes it inconvenient to use other than in an ad-hoc, hands-on manner.

Analog Devices, Inc. has adapted PCATTCP to produce a version that is more suited to use in an embedded environment and which has the added capability of  being able to stress-test the TCP/IP stacks with which it is used. It is a testament to the program’s clear structure and clean implementation that this has been achieved without wholesale rewriting.

# Program Structure

Rather than being a standard command line driven, one-shot program ADI-TTCP is structured as a server. It sits waiting to accept the connection of a TCP control stream on a specific port. From this stream it reads one or more transfer requests. Each request, which looks a lot like a set of  PCATTCP command line switches, specifies the parameters for a transfer: send or receive, TCP or UDP, total data size or total elapsed time, ‘record size’, target IP address, and so on. ADI-TTCP verifies each request as it receives it, performs some initial setup and sends a response via the control stream, including a transfer identifier. On receipt of a ‘run’ command on the command stream ADI-TTCP actions all of the transfer requests concurrently, starting a new thread for each. It records the start and stop time of each transfer and the processor usage during that time. Once the longest-running transfer has finished the results from each transfer, including the throughput value and processor usage figure, are returned via the control stream. The resources used by each test are released and ADI-TTCP awaits further commands from the control stream. If the controlling end closes the control stream then ADI-TTCP will sit waiting to accept a connection from another controller. Alternatively, a controller can send a ‘quit’ command to cause ADI-TTCP to terminate execution.

With this sort of structure ADI-TTCP can be used in a traditional ‘flat out’ mode to measure the throughput of a single connection with a given set of parameters, to stress test a TCP/IP stack by running multiple concurrent connections each with its own parameters, to simulate required data rates and measure their processor overheads, to soak test by running a transfer for many hours and to cycle through combinations of parameter values to record performance characteristics.

As described above, ADI-TTCP requires a controller of some description to send it commands and to receive results. Since the commands and results are text based, control could be provided using one instance of a telnet client (directed to ADI-TTCP’s control port) per instance of ADI-TTCP in the network under test but the expectation is that some sort of scripting language, possibly with a GUI front end, will be used. Tcl/Tk, Python, Lua and others all possess easy means of making TCP connections and manipulating text for constructing commands and analysing responses.

ADI-TTCP has the additional capability of acting as a ‘go-between’ for a remote controller that cannot communicate directly with instances of ADI-TTCP running on nodes of a local network. That is to say if the program is running on machine X which has two network interfaces, one connected to a ‘public’ network and the other to a private network of machines each also running ADI-TTCP, then a controller running on a remote machine can configure X’s instance of the program to relay its control commands to the machines on the private network and relay their response back to it. By this means throughput or stress testing that might involve gigabytes of data transfer can be controlled over a public network without the data transfers themselves having to take place over the same network.

# The Control Stream

ADI-TTCP accepts connections, one at a time, on a specific TCP/IP port (default 6000). It receives configuration commands, transfer descriptions and control commands over this connection and returns acknowledgements, transfer results and error messages. All traffic is text based and line oriented, with end-of-line being denoted by ‘\n’ (‘\r’ is ignored). Control traffic is low volume and none is generated or expected while network transfers are actually running. The control stream stays open during transfer execution, however, thus using up one socket and its attendant resources.

The following subsections describe each of the messages that a controller may send to ADI-TTCP and the responses that the program may return.

## Set up control stream relay

RELAY ip-address [port]

ip-address is a numeric internet protocol address in dot-notation.

port is the port number on which the instance of ADI-TTCP running at ip-address is listening for control stream connections. If omitted the default port number (6000) is assumed.

This command is best described by means of an example: machines X and Y are connected on one network and Y and Z on another. X and Z cannot ‘see’ one another yet X wishes to control the running of ADI-TTCP on both Y and Z. X sends Y a RELAY command specifying Z’s IP address and, optionally, the port number on which it accepts control stream connections. This command instructs Y  to create a TCP socket and listen for X  making a connection on it. When the connection is made Y  will in turn connect to Z’s command stream and will thereafter relay anything X sends it to Z’s control stream and Z’s responses back to X.

The response to the RELAY command that Y returns to X  is the port number (on Y ) to which X should connect in order to open Z’s control stream.

RELAY commands remain in force until the control stream on which they were issued is closed.

Note that executing a RELAY command is a bit costly in terms of open sockets and associated state. It is expected that the machines involved, X and Y in the example, would be resource-rich PCs rather than embedded platforms.

## Specify a transfer

TRANSFER –t –p port		# transmit to specified port on target
           [-u]		# use UDP instead of TCP
           [-l buflen]		# length of buffers written to network (1000)
           [-n numbufs]	# number of buffers to be written (20000)
           [-w delay]		# microseconds of delay before each write (0)
           [-L rate]		# desired transmit rate in bytes/sec (infinite)
           [-P period]		# number of seconds for which to transmit
           [-c]		# transmit continuously
           [-D]		# set TCP_NODELAY socket option if supported
           [-d]		# set SO_DEBUG socket option if supported
           [-b socksize]	# set SO_SNDBUF socket option if supported
           [-A align]		# align program’s buffer to this modulus (none)
           [-O offset]		# transmit from this offset from modulus (0)
           [-f fmt]		# format for reporting transmit rate (K)
           ip-addr		# target’s IP address

TRANSFER –r –p port		# listen on specified port
           [-u]		# use UDP instead of TCP
           [-l buflen]		# length of buffers read from network (1000)
           [-d]		# set SO_DEBUG socket option if supported
           [-b socksize]	# set SO_RCVBUF socket option if supported
           [-A align]		# align program’s buffer to this modulus (none)
           [-O offset]		# receive into this offset from modulus (0)
           [-f fmt]		# format for reporting receive rate (K)
           [-T]		# ‘touch’ each received data byte

Values shown in parentheses are the default values.

All simple numeric values are integers.

ip-address is a numeric internet protocol address in dot-notation.

fmt is a single character that specifies the units ADI-TTCP uses for reporting data throughput:
   K = kilobyte, k = kilobit, M = megabyte, m = megabit, G = gigabyte, g = gigabit     (per sec)
Note that ‘kilo’ is 1000, ‘mega’ is 1000000, etc.

A TRANSFER command describes one end of a data transfer. A controller describes the desired transmit characteristics to the instance of ADI-TTCP running on one machine, the receive characteristics to ADI-TTCP on another machine and issues a RUN command to both machines to execute the transfer.

The size of a transfer can be specified as the product of buflen and numbufs, the number of bytes transferred in period seconds and as the number transferred until a failure occurs ( [-c] ). If a period is specified then checking for expiry of the period takes precedence over counting the number of bytes or error checking. If option[-c]  is used then numbufs is ignored. Note that period is not a timeout: the ADI-TTCP program must be actively producing data for it to be able to check for expiration.

The response to a TRANSFER command is either an integer value or an error string prefixed with the characters ERROR: . The integer value is a transfer identifier which the RUN command will use to tag the transfer’s results.

## Run transfers

RUN

This command instructs ADI-TTCP to start concurrent execution of all the TCP and/or UDP transfers specified in previous TRANSFER commands.

When all the transfers are complete ADI-TTCP responds with a series of lines on the control stream. For each of the transfers it writes the lines:

- id START RESULTS	where id is the transfer identifier

- all the information lines produced by ADI-TTCP for the transfer, including throughput and processor load

- id END RESULTS

## Terminate ADI-TTCP

QUIT

The QUIT command causes ADI-TTCP to close all sockets, release all resources and terminate execution. If the controller closes the control stream without sending this command then ADI-TTCP cancels the effect of any REDIRECT commands and closes all sockets except for the one on which it listens for controllers. It then waits for a new controller to establish contact.

There is no response generated for the QUIT command.

# The ADI-TTCP Distribution

The ADI-TTCP distribution consists of pre-built binaries of the ADI-TTCP program, projects and  source code for building Windows and Blackfin(lwIP) versions and Python demonstration scripts. The subdirectories and their contents are:

- bin
This is where the build projects place their final executables and from where they should be executed.

- build\win32
This directory contains a Microsoft Visual Studio .NET  2003 solution file for building the Windows version of ADI-TTCP.

- build\vdk
In here are two VisualDSP++ 4.0 project files for building two copies of the ADI-TTCP program for the ADSP-BF537 EZ-Kit. Each copy is a VDK/lwIP program and they differ only in the values of the static IP address that is built into them (192.168.0.100 and 192.168.0.101). They require to be built using a version of VDSP++ no earlier than 4.0 + September 2005 Update. Also note that the programs will not run correctly on silicon revisions earlier than 0.2 and were tested on EZ-Kit revision 1.3. The EZ-Kits must have a valid MAC address burned in flash (see the EZ-Kit manual).

- build\lib
This contains versions of the lwIP and  driver libraries that contain performance enhancements which have not yet been incorporated into a VDSP++ Update.

- doc
This is where this document lives.

- scripts
This contains some Python modules and a demonstration script that show one way of controlling the execution of ADI-TTCP instances and displaying their results. See the following section for more details of the demonstration.

- src
The subdirectories within here contain the source code of ADI-TTCP and of the version of PCATTCP upon which it is based.  ADI-TTCP’s code is split into common and target-specific subdirectories.

ADI-TTCP’s common code makes calls on functions that must be implemented in target specific code for dealing with system dependant actions such as starting and stopping threads, sleeping, etc. See file common\adi_ttcp.h for a list of the functions that must be implemented.

Each target must provide a config.h file containing all target-specific #include file processing, typedef definitions, and whatever other manipulations are required to build the program.

# Demonstration Of Use

The scripts subdirectory of the ADI-TTCP distribution contains two Python scripts, ttcplib.py and ttcpcontrol.py, which contain classes and functions for creating control streams and handling TRANSFER and RUN commands. The former script is suitable for command line use and the latter in GUI environments. Script Demo.py creates a simple GUI in which the user can choose between three fixed test sets and view the results presented in a table. The three test sets are:

- Test peak throughput versus buffer size

- Test CPU use versus data rate

- Test CPU use versus number of streams

See  for an illustration of the GUI in action. The radio buttons at the lower left select between the available tests and the adjacent text area displays a short description of the chosen test. Clicking the large RUN TEST button starts the test and as each TRANSFER terminates its results are displayed in the appropriate quadrant of the table.

Figure 1  Demo GUI

The demonstration requires Python version 2.4 to execute it. It was developed and tested with the pre-built installation kit ActivePython-2.4.1-247-win32-ix86.msi available free of charge from www.activestate.com .

The demonstration GUI makes use of one non-standard widget, tktable. The ADI-TTCP scripts directory contains a distribution of tktable. To install it copy or move the Tktable2.9 subdirectory of scripts to the directory called tcl in your Python installation (C:\Python24\tcl in the ActiveState distribution, assuming the default location is used). This must be done before attempting to run Demo.py for the first time.

As supplied, the Demo.py script expects to control the running of ADI-TTCP on machines with IP addresses 192.168.0.100 (HOST1) and 102.168.0.101 (HOST2). ADI-TTCP must be already running on HOST1 and HOST2 and have displayed a message to the effect that they are waiting for a control stream connection on port 6000 before Demo.py is run, otherwise an error box will eventually appear and the demonstration will be abandoned.

Double-clicking Demo.py or issuing the command python Demo.py in a Command Prompt window makes the demonstration issue its TRANSFER and RUN commands directly to HOST1 and HOST2 so they need to be accessible from the machine on which Demo.py is running. In fact, it is envisaged that the target hosts will be on their own private subnet, probably using a hub, to avoid flooding a more public network. If the script’s host is not on the same subnet as the target hosts but another accessible PC is then the script can use a copy of ADI-TTCP running on that other PC to relay its commands to the target hosts and to relay their response back to the script. To achive this start the script using the command python Demo.py relay-host “text description of relay host” where relay-host is the network name or IP address of the PC doing the relay function and the text description argument is optional.

As well as the main ‘Control and Results’ window the demonstration script opens a ‘Logs’ window in which the control stream traffic between the script and the target hosts can be viewed. This window is split horizontally into two or three panes, one each for the transfer targets and one for the relay host if used. Closing the main window also closes the Logs window and exits the demonstration.

- PCAUSA’s Terms and Conditions

## http://www.pcausa.com/Utilities/ttcpdown1.htm

## Download

## PCAUSA Test TCP (PCATTCP) Executables And Source

Please read the Terms And Conditions and enter your Contact Information before downloading the PCAUSA Test TCP (PCATTCP) software.

You will not receive unsolicited e-mail from PCAUSA or any other party by entering your personal information in the form below. For more information, view the PCAUSA Privacy Policy.

### Terms And Conditions

THE UTILITIES PROVIDED BY Printing Communications Assoc., Inc. (PCAUSA) (THE "SOFTWARE") IS PROVIDED BY PCAUSA "AS IS" AND "WITH ALL FAULTS." PCAUSA MAKES NO REPRESENTATIONS OR WARRANTIES OF ANY KIND CONCERNING THE QUALITY,
SAFETY OR SUITABILITY OF THE SOFTWARE, EITHER EXPRESS OR IMPLIED, INCLUDING WITHOUT LIMITATION ANY IMPLIED WARRANTIES OF MERCHANTABILITY, OR FITNESS FOR A PARTICULAR PURPOSE. 

You are solely responsible for adequate protection and backup of the data and equipment used in connection with any of the Software, and PCAUSA will not be liable for any damages that you may suffer in connection with using, modifying or distributing any of the Software. 

No part of PCAUSA's Internet site(s) may be reproduced in any way, or by any means, without the prior written permission of PCAUSA.

You may distribute the PCATTCP software and its sources freely.