//This is a collection of design notes assembled during the development phase.

//These notes are informal by intention; but, contain many useful details

//to be considered along with the code comments.

cclk - 500 MHz (as configured by basic.crt auto-generated VDSP startup file)

sclk - 100 MHz (" ")

Program flag register function and directions

---------------------------------------------

Wrap TMR3 (PF6) [high drive pin] to PF15 (TIMERCLK) using EZ-KIT PPI header and jumper cable. Configure TMR3 for SCLK source (interim clock). Run TMR3 at 10 MHz. This results in TMRCLK = 10MHz.

This approach mimics an external oscillator input to the TMRCLK pin. The AD7656 CONVSTA input may require a rock solid timing reference input, in which case an external precision powered oscillator will drive the TMRCLK input from which CONVSTA will be derived.

Configure TMR2 (PF7) for PWM mode with external clock source (TIMERCLK). Divide by 162 (with proper duty cycle and polarity) and set timer counters for appropriate wave form duty cycle to yield 61728 Hz sampling. This signal will

drive the AD7556 CONVSTA input.

// AMS1 read of AD7656: 9 cycles for now. Can probably use 6 or 7.

// so w/SCLK=10MHz: 9*10*6 = 540 ns to read 6 channels.

// 61728 Hz allows for 8100 cclks between samples; so, without using a ping pong

// buffer scheme, roughly 7500 cclks are available for the FFT. A 256 pt FFT

// consumes 4465 cclks (as verified using the BF537 cycle counter).

CONVSTA can derive from a direct SCLK timebase as well. This approach could provide finer granularity sampling rate generation; but, it does not emulate an external oscillator which may be required for AD7656 sampling accuracy.

//Programmble Flag Configuration Notes:

(PF6)  O - alternate function TMR3     //will possibly be replaced by external

//precision oscillator

(PF15) I - alternate function TMRCLK

(PF7)  O - alternate function TMR2 [high drive pin]  //routed to AD7656 CONVSTA

//input via EZ-KIT

//uconnector

(PF2)  I - GPIO                    //start one-shot convert EZ-KIT

//pushbutton (IRQ signal)

(PF3)  O - GPIO [high drive pin]   //routed to AD7656 H/S_ input via EZ-KIT

//uconnector

(PF4)  O - GPIO [high drive pin]   //routed to AD7656 RESET input via EZ-KIT

//uconnector

(PF5)  I - GPIO             //start block convert EZ-KIT pushbutton (IRQ signal)

(PF8)  I - GPIO             //receives the AD7656 BUSY output via EZ-KIT

//uconnector(IRQ signal)

SW5 on EZ-KIT    BF537             BF537

-----             -----

pos1 - PF2_PB1 - PF2 - SW5_close - INPUT  //Start one shot button. Pressing

//button causes rising edge input to

//DSP

pos2 - PF3_PB2 - PF3 - SW5_open  - OUTPUT  //This signal is routed to the

//AD7656 H/S_ input via EZ-KIT

//uconnector

pos3 - PF4_PB3 - PF4 - SW5_open  - OUTPUT  //This signal is routed to the AD7656

//RESET input via EZ-KIT uconnector

pos4 - PF5_PB4 - PF5 - SW5_close - INPUT    //Start block convert button.

//Pressing button causes rising edge

//input to DSP

BF537 EBIU/AD7656 connections - routed via EZ-KIT uconnector

------------------------------------------------------------

D0:15

WR

RD

AMS1

AD7656 pulldowns:

-----------------

pin61: SER/PAR_

pin27: RANGE

pin22: CONVSTB

pin21: CONVSTC

pin29: W_/B

AD7656 pullups:

---------------

pin24: STBY

EZ-KIT switch settings:

-----------------------

Start with default ADSP-BF537 EZ-KIT switch settings. Then:

SW6: 1 closed, 2-4 open (AMS0: Flash connection)    //use AMS1 for AD7656 CS

SW7: 1-5 open                                       //disconnect audio interface

//from SPORT0

SW2: 1-4 open                                       //CAN/PF pins disconnected

SW3: don't care

SW8: don't care

SW1: don't care

SW4: don't care       //default UART connections OK per PF pin allocation above

connect p12-3 to p8-6 for clock jumper

--------------------- notes ------------------------

Port F, G interrupt A 27 IVG12 5

Port F    interrupt B 31 IVG13 6