| Engineer: | D. YOUNG | Part Number: | 600-00564-00 |
| --- | --- | --- | --- |
| PCB Designer: | VIOREL VADUVA | Revision: | A |
| MECH Designer: | N/A | Board Name: | PCB, CUSTOMER EVAL PLL & VCO 2XFO |
| Checker: |  | Review Date: |  |
| ECR Number: |  | Charge Number: | 2012626 |

| Designer | Checker | N/A | Layout Verification |  |
| --- | --- | --- | --- | --- |
|  |  |  | Spacing and Connectivity |  |
|  |  |  | Impedance Control Rules | Top Side 0.016” 50 ohms |
|  |  |  | Power/GND Planes | PWR current capacities/net verification |
|  |  |  | VIA types | Blind/Back drilled/FILLED/HOT VIAS/ Mask for MTG vias |
|  |  |  | GND VIAS | RF Shielding/Cover wall centers/antennas |
|  |  |  | IC center GND slugs | Vias placed/Soldermask/Pastemask copper |
|  |  |  | Printed Components | Layer 20/Soldermask/Keep outs |
|  |  |  | Unused pads | If removing-preserve via pads on start/end layers |
|  |  |  | Soldermask | 10mil solder dam/no silkscreen in solderable area |
|  |  |  | Silkscreen | Polarities/Board #/HMC logo/Connector labels |
|  |  |  | Fiducials | Omit Ref Des from silkscreen/add to stencil |
|  |  |  | Mechanical |  |
|  |  |  | Cover Wall | Width/keep out/soldermask size |
|  |  |  | Cover & Mounting holes | Sizes and locations |
|  |  |  | Critical comp. locations | Connectors/Dimensions/Bottom layer clearances |
|  |  |  | Keep-out areas | Verify keep outs for components, VIAS, GND pour, etc |
|  |  |  | Solder-mask openings | Screws-washers/Bottom around connectors |
|  |  |  | DXF Verification |  |
|  |  |  | Fabrication |  |
|  |  |  | Specify IPC standard | 6012 CLASS 2 |
|  |  |  | PCB Dimensions | Overall/cutouts/radius/angles/edge plating/coupons |
|  |  |  | Layer stack-up detail | Via cross-section/Layers/Thickness as Critical Dimension |
|  |  |  | Material, copper weight | Stack-up to match notes |
|  |  |  | Finish plating req. | Multi-finish/Wire-bonding/OSP |
|  |  |  | Special requirements | Impedance-line widths/Blind/Back drilled vias/Edge pltg |
|  |  |  | VIA fill requirements | Shown in cross-section detail and/or in notes |
|  |  |  | Minimum annular ring | VIAS and thru holes |
|  |  |  | Hole Chart | Match drill files/Partials/Merge drill sizes/Tolerances |
|  |  |  | Gerber, Drill Data |  |
|  |  |  | Artwork, output files | Appropriate labels & RS-274X output format) |
|  |  |  | Printed Components | Proper size shapes and gaps/Metal-01 layer 20 |
|  |  |  | Soldermask | Oversize 8 mils/POSITIVE Image-must be clearly labeled |
|  |  |  | Stencil/Paste-mask | Undersize 2 mils/omit printed components |
|  |  |  | NCDrill | 3 files for each type: .rep, .drl, .lst |
|  |  |  | GC-Prevue file | Engineer check/Design Review |
|  |  |  | ZIP file | Readme.txt/output files/fab pdf/IPC Netlist/aperture.rep/ASCII file/Component XY/GC-Prevue/copy of this checklist |
|  |  |  | Assembly Dwg |  |
|  |  |  | Ref Des readable |  |
|  |  |  | Notes |  |
|  |  |  | Detail Views |  |