### 4/4/2000

DC297 QUICK START GUIDE

Connect a 5VDC input source capable of at least 7.5 amps to the input terminals.  If the power sources are adjustable, be sure to set the output voltages correctly, before connecting the supply to the demo board.  At high output load current, it may be necessary to increase the output of the 5 volt supply to maintain the 5V at the input terminals, voltage drops on input cables can be substantial.

C9 is rated at 6VDC, by changing it to a 15VDC cap input voltage may be raised to 12VDC.

Use caution when connecting a load to the output, these voltages are LETHAL.

The test load may consist of power resistors, or a suitable electronic load box.

If resistors are used be sure their power rating is adequate, this demo board can output up to 45 watts of power. Again, the output voltages are lethal.

Solder connections securely, clip leads for power connections are not recommended.  Once all connections are secure, turn on the input power source.

Verify input and output voltages at the terminals.

There are no bleeder resistors on the output, so the board will remain charged

for some time after power is removed.

The board will operate at less than 5VDC  as low as 3.6VDC however caution must be used not to overload the output causing the input voltage to sag.  In some

systems that use a microprocessor care must be taken not to overload the plus and minus outputs as this could cause the 5VDC supply to sag sufficiently to reset the

microprocessor.

Al Guizar

LTC Design Center

Colorado Springs, Colorado

719 262-9218

ralf@bwn.net