# Description

The DC192 is a collection of subcircuits to support the LT1684 in ring signal generation.

This board contains the hardware to facilitate demonstration of the LT1684 in several different system modes of operation.  For ease of use the DC192A is designed to operate from a single 5V input supply.

It utilizes the popular Microchip PIC16C73 microcontroller for the generation of the ring signals, cadence, zero crossing output, connect output, and various other specialized features detailed further in the Step by

# Setup 1 Generic mode

Perhaps the most generic setup for the DC192A is the 20Hz 3 REN Cadenced ring. This is the setup the Demo Circuit is shipped with.

In the generic mode the board is configured to demonstrate its ability to ring a phone.

- Insure that the jumpers configure the board as setup 1.

- Connect +5V and Gnd to the board via J2 (center+) or TP3 +5V and TP5 Gnd.

- Plug phone into J1 or attach Tip to TP2 and Ring to TP1. Or

- Attach oscilloscope ground lead to TP1 and the probe to TP2.

- Power up the board; observe that LED1 and LED4 are illuminated.

- Depress SW1 and  observe the results:

- A cadenced ring signal appears at the output to be seen and/or heard.

- LED2 illuminates to indicate the connect condition while TP13 (RELAY) is pulled to ground. (Remember this board is isolated so the scope ground on TP1 is not connected to the input ground on TP5, the RELAY signal is pulled to the TP5 ground.)

- A short pulse appears on TP14 (ZERO) a few milliseconds prior to each zero crossing.

- LED4 twinkles as the duty factor of its drive is modulated with the 20Hz ring signal.

The effect of holding SW1 down continuously can be obtained by emplacing jumper 14 to short its terminals, or by shorting TP4 (CADENCE) to ground (TP5).

## Other Setups

At present other setups are best described in the jumpers file (appended to the end of this document).

As other modes of operation are incorporated this document, and the firmware will be updated to implement, and document the new modes.

JUMPER CONFIG	OCT 7 99

JP10 IS CW/CADENCED SELECT IN = CW OUT = CADENCED

WHEN IN:

IF 	1) 	JP14 IS IN OR

2)	SW1 IS DEPRESSED OR

3)  	CADENCE IN IS PULLED LOW

THEN	CONTINOUS SINEWAVE IS OUTPUT.

ELSE	CONTINOUS ZERO IS OUTPUT

WHEN OUT:

IF 	1) 	JP14 IS IN OR

2)	SW1 IS DEPRESSED OR

3)  	CADENCE IN IS PULLED LOW

THEN	CADENCED SINEWAVE IS OUTPUT

ELSE	CONTINOUS ZERO IS OUTPUT

JP12 IS CONNECT RELAY MODE IN = TEST OUT = NORMAL

WHEN	JP12 IS IN AND

JP10 IS IN AND

IF 	1) 	JP14 IS IN OR

2)	SW1 IS DEPRESSED OR

3)  	CADENCE IN IS PULLED LOW

THEN	CONTINOUS SINEWAVE IS OUTPUT.

THE CONNECT RELAY (TP13) IS ENERGIZED EVERY EIGHTH CYCLE

ELSE	CONTINOUS ZERO IS OUTPUT

THE CONNECT RELAY (TP13) IS DE-ENERGIZED CONTINOUSLY

WHEN	JP12 IS IN AND

JP10 IS OUT AND

IF 	1) 	JP14 IS IN OR

2)	SW1 IS DEPRESSED OR

3)  	CADENCE IN IS PULLED LOW

THEN	CADENCED SINEWAVE IS OUTPUT.

THE CONNECT RELAY (TP13) IS ENERGIZED EVERY EIGHTH CADENCE

ELSE	CONTINOUS ZERO IS OUTPUT

THE CONNECT RELAY (TP13) IS DE-ENERGIZED CONTINOUSLY

WHEN	JP12 IS OUT AND

JP10 IS IN AND

IF 	1) 	JP14 IS IN OR

2)	SW1 IS DEPRESSED OR

3)  	CADENCE IN IS PULLED LOW

THEN	CONTINOUS SINEWAVE IS OUTPUT.

THE CONNECT RELAY (TP13) IS ENERGIZED CONTINOUSLY

ELSE	CONTINOUS ZERO IS OUTPUT

THE CONNECT RELAY (TP13) IS DE-ENERGIZED CONTINOUSLY

WHEN	JP12 IS OUT AND

JP10 IS OUT AND

IF 	1) 	JP14 IS IN OR

2)	SW1 IS DEPRESSED OR

3)  	CADENCE IN IS PULLED LOW

THEN	CADENCED SINEWAVE IS OUTPUT.

THE CONNECT RELAY (TP13) IS ENERGIZED EVERY CADENCED CYCLE

ELSE	CONTINOUS ZERO IS OUTPUT

THE CONNECT RELAY (TP13) IS DE-ENERGIZED CONTINOUSLY

JUMPER 6:	SOFTSTART AND UNDERVOLTAGE LOCKOUT DISABLE

WHEN 	JP6 IS IN	THERE IS NO SOFTSTART OR UNDERVOLTAGE LOCKOUT\

JP6 IS OUT 	SOFTSTART STARTS @ 4.75V AND

UNDERVOLTAGE LOCKOUT SHUTS DOWN AT 4.5V

JUMPERS 4 & 5	HIGH OUTPUT CURRENT / LOW OUTPUT CURRENT

WHEN 	JP4&5 ARE IN 	THE OUTPUT CAPACITY OF THE DEMO BOARD IS 10 REN

JP4&5 ARE OUT	THE OUTPUT CAPACITY IS 1 REN

PATH CONFIGURATION JUMPERS:

JUMPER 15	PWM SOURCE SELECTOR

WHEN	JP15 IS BRIDGING 1&2	INTERNAL PWM IS SELECTED

JP15 IS BRIDGING 2&3	EXTERNAL PWM IS SELECTED VIA TP8

JUMPERS 1&2&3	REFERENCE MODE JUMPERS

THE ORIGIONAL SOFTWARE 10/11/99 DOES NOT SUPPORT VARIABLE REFERENCE MODE

SO THE JUMPERS MUST BE PLACED AS FOLLOWS:

JP1 MUST BRIDGE PINS 2&3

JP2 MUST BRIDGE PINS 2&3

JP3 MUST BRIDGE PINS 1&2

UNUSED JUMPERS:

JP7

JP8

JP9

JP11

JP13

UNUSED I/O PORTS:

BNC		ANALOG IN

J4		RS232 IN

TP10,11,12	SMBUSS

FOR TEST SET JUMPERS AS FOLLOWS

JP1	BRIDGE PINS 2&3

JP2	BRIDGE PINS 2&3

JP3	BRIDGE PINS	1&2

JP4 	INSTALLED

JP5 	INSTALLED

JP6	OPEN

JP7	OMIT

JP8	OMIT

JP9	OMIT

JP10	OPEN

JP11	OMIT

JP12	OPEN

JP13 	OMIT

JP14	INSTALLED

JP15	BRIDGE PINS 1&2

APPLY 5V BY J2 (+ IN CENTER)

LOOK AT THE LEDS TWINKLE