Keith Szolusha           Linear Technology Corporation               September 2001

# Demo Board DC436A Test Procedure

The board is a 1.25MHz, 6.5 to 25V input to 5V output buck regulator circuit @2A with all ceramic capacitors and using the SO8 package LT1765.  The minimum switch current limit is rated at 3A for the IC.  There are pins for Shutdown and Synchronization options as shown in the figure below.  They can be left floating when not in operation.  On the backside of the board, there is an open space to add a TAJ tantalum C case capacitor between the input pins for input transient damping, and there is a space between the output pins to add a small ceramic capacitor for further output ripple reduction or testing output ripple at the turret mounts.  These extra capacitors need not to be stuffed for typical operation of the board.  In customized operation, D3 should be used when the output voltage is set below 2.7V and D2 should be used when the output voltage is greater than 2.7V.  When one diode is being used, the other diode can be left unstuffed.

Use an adjustable 3A/30V supply, a 5A load box, and 4 Fluke meters.  Connect the circuit as shown above.  The short circuit condition is easily created by removing the load box and replacing it with a short circuit

Set and check the following parameters:

| Parameter | Condition | Min | Measured | Max | Units |
| --- | --- | --- | --- | --- | --- |
| Vout | Vin = 24V Iout = 500mA | 4.85 |  | 5.15 | V |
| Iin | Vin = 24V Iout = 500mA |  |  | 139 | mA |
| Vout | Vin = 24V Iout = 1.200A | 4.85 |  | 5.15 | V |
| Iin | Vin = 24V Iout = 1.200A |  |  | 334 | mA |
| Vout | Vin = 12V Iout = 500mA | 4.85 |  | 5.15 | V |
| Iin | Vin = 12V Iout = 500mA |  |  | 279 | mA |
| Vout | Vin = 12V Iout = 2.000A | 4.85 |  | 5.15 | V |
| Iin | Vin = 12V Iout = 2.000A |  |  | 1.115 | A |
| Vout | Vin = 8.0V Iout = 2.000A | 4.85 |  | 5.15 | V |
| Iin | Vin = 8.0V Iout = 2.000A |  |  | 1.672 | A |
| Vout | Vin = 7.0V Iout = 1.5A | 4.85 |  | 5.15 | V |
| Iin | Vin = 7.0V Iout = 1.5A |  |  | 1.379 | A |
| Vout | Vin = 12V Vout = Short Circuit | 0 |  | 250 | mV |
| Iout | Vin = 12V Vout = Short Circuit | 2 |  | 3 | A |
| Vout | Vin = 12V /Shdn = Gnd Rload = 100ohm | 0 |  | 0 | V |
| Iin | Vin = 12V /Shdn = Gnd Rload = 100ohm | 15 |  | 60 | A |
| Iout | Vin = 12V /Shdn = Gnd Rload = 100ohm | 0 |  | 0 | mA |